Release notes TNT V1.1alpha1 with AX25-kernel interface

-------------------------------------------------------

0. Preface

As TNT is using the AX25-kernel-interface and some functions of the

ax25-utilities, you must install all these stuff and get it working.

For help see the AX25-HOWTO. If you don't succeed, it makes NO sense to

try it with TNT!

As TNT is not using all features of the AX25-kernel (no NETROM, no ROSE)

and may use the other features incorrectly I consider it as status

alpha. The development was done using Kernel 2.0.28 and ax25-utilities

2.0.12c without further patches. Any other configuration was not tested and

may lead to problems. But I am interested in feedback of any kind to

improve the code.

In addition not all features of TNT in the hostmode-version are working

correctly. For example the PACSAT broadcast transmitter is broken. Most

other things should work.

Please read the documentation of TNT, as this is needed for proper operation

of the program.

1. Preparations

1.1 Library of ax25-utilities

TNT needs the library functions provided by the ax25-utilities. Therefore

you need to have a compiled version of the ax25-utilities on your system.

In addition you have to provide a symbolic link from the directory where

the tnt source is located to the 'lib'-directory of the ax25-utilities.

Suppose your tnt-dir is /usr/local/src/tnt1.1/ and your ax25-utilities-dir

is /usr/local/src/ax25-utils-2.0.12c, the following commands have to be

issued:

cd /usr/local/src/tnt1.1/

ln -s /usr/local/src/ax25-utils-2.0.12c/lib lib

1.2 Makefile of TNT

To compile TNT with AX25 kernel support, you have to adapt the Makefile.

Be sure to uncomment the three lines below:

# uncomment, if you want to use the Linux AX25 Kernel

CFLAGS          := $(CFLAGS) -DUSE_AX25K

LIBS            := $(LIBS) -lax25

LDFLAGS         := $(LDFLAGS) -Llib

1.3 special entries in 'tnt.ini'

1.3.1 ax25k_port

To use the AX25-kernel-interface in TNT, you have to define 'ax25k_port' in

'tnt.ini'. For example the line

ax25k_port P3

defines the use of the AX25-kernel and that P3 (a valid port from 'axports')

will be the default port.

The parameters concerning serial interface will be ignored, so they can have

every value.

1.3.2 fullmoni_flag

The kernel provides two ways of getting frames for the monitor screen.

a. Only received frames are displayed.

b. All frames are displayed.

The disadvantage of b. is that TNT has to analyse ALL frames of the network

including ethernet-traffic. So on a busy ethernet TNT will generate high

load because of this. So it depends on your configuration whether you can

use this option. To select the options you must use the parameter

'fullmoni_flag' in tnt.ini:

fullmoni 0      -> a.

fullmoni 1      -> b.

Generally monitoring is available only when TNT is started by root. If

started by a user the monitor is disabled. The kernel does not allow a user

to open a monitor socket.

1.3.3 tnt_errlog

All problems which will occur and may lead to a termination of TNT will be

stored in a logfile. The default filename is 'tnterr.log' in the main TNT

directory. If you want to define another name use parameter 'tnt_errlog'

in tnt.ini.

2. Some things on operation

2.1 Port handling

TNT reads the 'axports'-file at startup. It takes the definitions from there

and the portnames at the start of the lines will be taken as port prefixes.

You can check the assignment with the command 'QRG'.

The port prefixes (with a ':' at the end) define in all commands which port

shall be used.

If for example you have defined a port named 'P3' the command

'c p3:te1st' will try to connect te1st on port P3.

2.2 Callsign handling

At startup TNT will set the callsign of all channels to the callsign of the

default port defined in 'ax25k_port' in tnt.ini. This means, that TNT is

listening on all ports in 'axports' with the callsign of the default port.

This can be changed by assigning different callsigns to the channels using

'MYCALL' command. Currently the callsign defined is listening on all ports,

there is no way to just listen on only one port out of your ports defined in

'axports'.

If you temporarily want to use a different callsign/SSID you can use the

'I'-command as this will not fix the callsign on this channel. After a

disconnect the old callsign will be restored.

2.3 Unproto handling

The destination for UNPROTO frames is given via 'C'-command in channel 0,

the unproto channel. After startup UNPROTO frames will be transmitted to

'TNT' on the default port. If you give 'C P4:beacon', the UNPROTO frames

will contain the destination 'BEACON' and will be transmitted on port P4.

2.4 Parameters

The parameters normally handled by 'axparms', 'axctl' and 'kissparms' can be

handled by TNT directly. The use of the other programs is possible, too.

The parameters can be changed and displayed. KISS-parameters can be changed

only as there is no way to read the current values.

If you handle parameters on an unconnected channel, the default values of

the port are handled. If you don't give a port, the default port is used.

Examples: 'n 20' will set the retries on the default port to 20.

'o P3:6' will MAXFRAME to 6 on port P3

'@T2' will display the T2-timer on the default port

'@T3 P2:' will display the T3-timer on port P2

On a connected channel you will change the parameters of the connection on

this channel. After disconnect these values will be lost. If you don't give

a port, the port of the connection will be used. If a port different than

the port of the connection is given, the command is rejected.

Here is a list of the implemented parameters:

TNC-like   verbose     AX25-kernel-    KISS         value       special

command    command     value           value        readable    values

-------    --------    ------------    ---------    --------    -------

F          FRACK       T1              -            yes

N          RETRY       N2              -            yes

O          MAXFRAME    WINDOW          -            yes

P          PERSIST     -               PERSIST      no

R          DIGIPEAT    DIGI            -            yes           0-3

T          TXDELAY     -               TXDELAY      no

W          SLOTTIME    -               SLOTTIME     no

@D         FULLDUP     -               FULLDUP      no

@T2        RESPTIME    T2              -            yes

@T3        CHECK       T3              -            yes

@TA        -           -               TXTAIL       no

#BACKOFF   -           BACKOFF         -            yes           L/E

#IDLE      -           IDLE            -            yes

#PACLEN    -           PACLEN          -            yes

#HARDWARE  -           -               HARDWARE     no           0-255

(Hint: TNC-like commands without a verbose command have to be given with

'tnc' at the beginning: 'tnc #BACKOFF P3:')

To setup specific values at startup of TNT you can use 'tnt.up'. Be sure

to define a channel with the 'channel' command.

3. Credits

Thanks to all who worked on the AX25-Kernel and the ax25-utilities, as this

work is the basis of the code in 'ax25k.c'.

Mark Wahl, DL4YBG

(DL4YBG @ DB0GR.#BLN.DEU.EU)

(wahlm@berlin.snafu.de)

(http://www.snafu.de/~wahlm)